Theory

VHDL is an acronym for VHSIC Hardware Description Language (VHSIC stands for Very High

Speed Integrated Circuits). It is an IEEE industry-standard hardware description language

that can be used to model a digital system at many levels of abstraction, ranging from the

algorithmic level to the gate level. Unlike conventional programming languages, VHDL allows

designers to model how signals flow through logic gates and flip-flops concurrently (at the

same time).

VHDL Structure

A VHDL design is typically composed of three essential parts:

1. Library and Packages: These contain predefined data types, constants, and functions.

The most common library used is IEEE, which provides the standard std logic types.

2. Entity: Defines the external interface of the circuit. It specifies the input and output

ports (pins) through which the circuit communicates with the outside world.

3. Architecture: Defines the internal logic or behavior of the entity. It describes how the

outputs are generated based on the inputs.

Computer Architecture (CMP 262) Lab 1

Library

Entity

Architecture

Main Code

Includes the required libraries and packages, i.e. ieee, std, and work.

Specifies mainly the circuit’s I/O ports

(pins).

Defines the functionality of a digital cir-

cuit using the input and output “pins”

listed in the entity.

Libraries and Packages

A Library is a compiled directory where VHDL design units (such as entities, architectures,

and packages) are stored. The two most important libraries are:

• Library std: This is automatically included in every VHDL design. It contains the most

basic built-in data types such as bit, integer, boolean, and character, along with their

corresponding operators (logic, arithmetic, comparison, shift, and concatenation).

• Library IEEE: The most widely used library in VHDL. It provides the std logic and

std logic vector types that are essential for real hardware design. You must explicitly

declare this library and the relevant package at the top of your file:

library IEEE ;

use IEEE . STD_LOGIC_1164 .ALL ; -- Provides std_logic types

use IEEE . NUMERIC_STD .ALL ; -- Provides arithmetic on

std_logic

Entity

The entity describes the external view of a circuit, what goes in and what comes out. Think of

it as the “black box” definition of your component. It declares the ports: the names, directions

(in, out, inout), and data types of every signal that crosses the boundary.

The general syntax of an entity is:

entity < entity_name > is

port (

< port_name > : < direction > < data_type >;

< port_name > : < direction > < data_type >

-- No semicolon on the last port

) ;

end entity < entity_name >;

Port Directions:

• in: signal flows into the entity (input).

• out: signal flows out of the entity (output).

• inout: signal can flow both ways (bidirectional, e.g., a bus).

Example: An entity for a 2-input AND gate:

Computer Architecture (CMP 262) Lab 1

entity AND_GATE is

port (

A : in std_logic ;

B : in std_logic ;

Y : out std_logic

) ;

end entity AND_GATE ;

Architecture

The architecture defines the internal behavior or structure of the entity. If the entity is the

black box, the architecture is what is inside it. One entity can have multiple architectures (e.g.,

one behavioral and one structural), but only one is active at a time.

The general syntax of an architecture is:

architecture < arch_name > of < entity_name > is

-- Declarations : internal signals , constants , components

begin

-- Concurrent statements describing the logic

end architecture < arch_name >;

Example: Architecture for the AND gate:

architecture Behavioral of AND_GATE is

begin

Y <= A and B ; -- Concurrent signal assignment

end architecture Behavioral ;

A key concept here is that the statement Y <= A and B; is a concurrent signal assignment.

It does not execute line by line like a program, instead it continuously monitors A and B and

updates Y whenever either changes, just like a real logic gate.

Types of Architectural Models

VHDL supports three styles of writing an architecture, each describing the same circuit at a

different level of abstraction:

1. Behavioral Model: Describes what the circuit does using sequential statements inside

a process block. It is the most abstract style and closest to conventional programming.

architecture Behavioral of AND_GATE is

begin

process (A , B )

begin

Y <= A and B ;

end process ;

end architecture Behavioral ;

2. Dataflow Model: Describes how data flows through the circuit using concurrent signal

assignments (no process block). It is between behavioral and structural in abstraction

level.

Computer Architecture (CMP 262) Lab 1

architecture Dataflow of AND_GATE is

begin

Y <= A and B ; -- Executes concurrently , not sequentially

end architecture Dataflow ;

3. Structural Model: Describes the circuit as an interconnection of sub-components,

similar to a netlist or schematic. It is the most detailed and closest to the actual hardware.

architecture Structural of AND_GATE is

component AND2 -- Declare a lower - level component

port (X , Z : in std_logic ; W : out std_logic ) ;

end component ;

begin

U1 : AND2 port map ( X = > A , Z = > B , W = > Y ) ;

end architecture Structural ;

In this lab, we will primarily use the Dataflow style for its simplicity. Behavioral and Struc-

tural modeling will be explored in later labs.

Basic Data Types and Signals

1. std logic: The standard type for a single-bit signal. Unlike the simpler bit type (which

can only be ’0’ or ’1’), std logic supports nine values including ’Z’ (High Impedance)

and ’U’ (Uninitialized), making it much more realistic for hardware modeling.

2. std logic vector: Used to represent a group of bits (a bus). For example, std logic vector(7

downto 0) represents an 8-bit bus. The range 7 downto 0 means bit 7 is the most sig-

nificant bit (MSB).

3. Signals: Internal wires inside an architecture. They are declared between the architecture

... is line and the begin keyword.

architecture Behavioral of MY_CIRCUIT is

signal internal_wire : std_logic ; -- 1 -bit wire

signal data_bus : std_logic_vector (7 downto 0) ; -- 8 bit bus

begin

-- ...

end architecture Behavioral ;

VHDL Design Cycle

To verify a design before it is physically implemented, it must pass through the simulation

design cycle:

1. Analysis (Compilation): The compiler checks the source code for syntax errors and

ensures it follows language rules.

2. Elaboration: The hierarchical design is “flattened” into a netlist. The simulator binds

Computer Architecture (CMP 262) Lab 1

3. Simulation: Stimuli (inputs) are applied to the design to verify its functional behavior

over time.

4. Visual Verification: Tools like GTKWave read the generated VCD (Value Change

Dump) file to display the timing diagrams of the signals.

Worked Example: Buffer Design and Simulation

A buffer is the simplest possible circuit — it has one input and one output, and the output

simply follows the input. It is ideal as a first VHDL program because it lets you focus entirely

on the toolchain workflow without any gate logic getting in the way.

Step 1: Write the Design File (buffer.vhd)

Create a new file called buffer.vhd and type the following:

-- Library declarations ( always required for std_logic )

library IEEE ;

use IEEE . STD_LOGIC_1164 .ALL ;

-- Entity : one input , one output

entity MY_BUFFER is

port (

A : in std_logic ;

Y : out std_logic

) ;

end entity MY_BUFFER ;

-- Architecture : output simply follows input

architecture Dataflow of MY_BUFFER is

begin

Y <= A ;

end architecture Dataflow ;

Step 2: Write the Testbench (buffer tb.vhd)

A testbench is a special VHDL file used only for simulation. It has no input or output ports

(it is a closed system). It creates an instance of the design under test (DUT) and applies input

stimuli to observe the outputs.

library IEEE ;

use IEEE . STD_LOGIC_1164 .ALL ;

-- Testbench has NO ports

entity BUFFER_TB is

end entity BUFFER_TB ;

architecture Simulation of BUFFER_TB is

-- Signals to connect to the DUT

signal tb_A : std_logic := ’0 ’;

signal tb_Y : std_logic ;

Computer Architecture (CMP 262) Lab 1

begin

-- Instantiate ( connect ) the Design Under Test (DUT )

DUT : entity work . MY_BUFFER

port map (

A = > tb_A ,

Y = > tb_Y

) ;

-- Stimulus process : toggle the input

STIMULUS : process

begin

tb_A <= ’0 ’; wait for 10 ns ; -- Y should follow : ’0 ’

tb_A <= ’1 ’; wait for 10 ns ; -- Y should follow : ’1 ’

tb_A <= ’0 ’; wait for 10 ns ; -- Y should follow : ’0 ’

wait ; -- End simulation

end process STIMULUS ;

end architecture Simulation ;

Step 3: Compile, Elaborate, and Simulate with GHDL

Open a terminal in the folder containing both .vhd files and run the following commands in

order:

# 1. Analyze ( compile ) both files

ghdl -a buffer . vhd buffer_tb . vhd

# 2. Elaborate ( link ) the testbench

ghdl -e buffer_tb

# 3. Simulate and export waveform to a VCD file

ghdl -r buffer_tb -- vcd = simulation . vcd

If there are no errors, a file called simulation.vcd will be created.

Step 4: View the Waveform in GTKWave

# Open GTKwave for waveform

gtkwave simulation . vcd

In the GTKWave window:

1. In the left panel (SST), expand BUFFER TB and click on Simulation.

2. Select tb A and tb Y by holding Ctrl and clicking, then click Append.

3. Press Ctrl+Shift+F (or click Time → Zoom to Fit) to see the full waveform.

4. Verify that tb Y is always identical to tb A at every point in time.

Computer Architecture (CMP 262) Lab 1

Step 5 (Optional): Automate with run.bat

Once you are comfortable with the individual steps above, you can automate Steps 3 and 4

using the provided run.bat script. Save the following as run.bat in your project folder:

@echo off

set DESIGN =%1

set TB =%2

ghdl -a % DESIGN %. vhd % TB %. vhd

ghdl -e % TB %

ghdl -r % TB % -- vcd = simulation . vcd

gtkwave simulation . vcd

Then run the entire workflow with a single command:

run buffer buffer_tb

The script takes two arguments: the design filename (without .vhd) and the testbench

entity name. You can reuse this script for every future lab, just change the arguments.